H 8 0 6 S A

Ⅰ.Introduction

H806SA has Four outputs, supports maximum 4096 pixels, it Support Art-Net protocol and can work with DMX console or set address for DMX chips .H801SA also can add or delete DAT files through the LAN.

Ⅱ. Features

- Eight output ports, controls maximum 4096 pixels, each port controls maximum 1024 pixels.

- SD card supports FAT32, FAT16 format; maximum capacity is 64G bytes, stores up to 64 DAT files.

- H806SA can be connected to DMX console and occupies 6 or 7 channels, so you can switch files, adjust speed and brightness with DMX console.

- It supports art-net protocol, and can directly use software supporting artnet such as MARIX and JINX.

- Support encryption function of limiting power on times.

- Support the address of the DMX chip, with the function of several points.

- It can be connected and detachable, can be directly connected to the computer network card, router, or to the main controller.

- Support online firmware upgrade.

- Support for DMX512_ RDM read back.

Ⅲ. Instructions

1. The SD card needs to be formatted into FAT32 or FAT16 format after adding and deleting files for many times. Support SDHC (High Capacity SD card), the maximum is 64g bytes. Up to 64 DAT files are allowed in the SD card, and the playback order is sorted by letter according to the file name.

2. Four output ports, up to 4096 pixels. The maximum number of control points per port is 1024. It can also be set as one or two output ports. Each port can control 2048 points at most. Ports three or four are backup of port one and two respectively. All ports are differential outputs.

3. Power on the nixie tube to display the version number, press and hold the "menu" key and power on again, the controller automatically upgrades the firmware.

4. Press the menu key to switch the menu, press the '-' or '+' key for setting, long press the '-' or '+' key to speed up the parameter setting, and press the OK key to save the parameters or enter the submenu.

① After power on, the file menu will be displayed, the first nixie tube will display F, and the document number will be displayed at the back.

② Press the menu key to switch to the speed menu s in the range of 1 – 100 frames per second. When the power supply is synchronized, the speed of each controller must be the same.

③ The next menu is brightness B, the range is 0-16.

④ The next menu is the start channel address a of the DMX.

When the range is 1-512 and the number of color components is 4, each controller occupies 8 channels, otherwise each controller occupies 7 channels. The default starting address is 1, i.e. channel 1-7 is occupied. If the starting channel address is 2, channels 2-8 are occupied.

The first channel is brightness, the luminance channel value 0-3 corresponds to 0 (not bright), 4-7 corresponds to 1, 8-11 corresponds to 2 252-255 corresponds to 63.

Channel 2 is the red component.

The third channel is the green component.

Channel 4 is the blue component.

When the number of color components is less than 4, 7 channels are occupied

Channel 5 is the dat file number. The relationship between the DMX512 console channel values and the corresponding files is: four values correspond to one file, that is, 0-3 corresponds to the first file, 4-7 corresponds to the second file 252-255 corresponds to the 64th file.

Channel 6 is the playback speed. The relationship between the corresponding speed is (speed set by the key) multiplied by (speed channel value) and divided by 255, that is, the maximum value of 255 on the console corresponds to the speed set by the key.

Channel 7 is direction and strobe, 0-123 is forward, 124-131 is stop, 132-255 is reverse. 64-127 is forward stroboscopic, 192-255 is reverse stroboscopic, others are not stroboscopic.

When the number of color components is equal to 4, 8 channels are occupied

Channel 5 is the white component.

The sixth channel is the dat file number. The relationship between the DMX512 console channel values and the corresponding files is, four values correspond to one file, that is, 0-3 corresponds to the first file, 4-7 corresponds to the second file 252-255 corresponds to the 64th file.

Channel 7 is the playback speed. The relationship between the corresponding speed is (speed set by the key) multiplied by (speed channel value) and divided by 255, that is, the maximum value of 255 on the console corresponds to the speed set by the key.

Channel 8 is direction and strobe, 0-123 is forward, 124-131 is stop, 132-255 is reverse. 64-127 is forward stroboscopic, 192-255 is reverse stroboscopic, others are not stroboscopic.

⑤ The next menu is IP address P. the first three bytes are adaptive.

There cannot be the same IP address in the same LAN. Long press "OK" to enter the artnet submenu, and then press "menu" to switch the submenu:

I. set the number of sub controllers in artnet, and how many sub controllers are connected in this way.

II. Set the chip models controlled by artnet, including DMX512, tm1812, tm1914, tm1814 and qed3110.

Set the starting space and change the sub control number of offline control.

IV. how many channels can a port carry

Then press the menu key to return to the main menu.

⑥ The sixth menu is to set the number of color components n of a point. The default value is 3, that is RGB. The range is 1-4.

Press and hold "OK" to enter the address submenu of DMX chip, and then press "menu" to switch the submenu:

I. first, select the dmx512ap, sm1651x, sm1751x, sm1751x, sm1751x, sm1751x, sm1751x, sm1751x, sm1751x, sm1751x, C12, C12, C13, respectively.

Ⅱ.Set the number of channels occupied by a lamp or transcoding board. If it is a point light source, this value is the same as the number of color components n. if C05 is selected for the DMX chip, P is displayed here, indicating the number of points carried by transcoding.

III. press the menu key to set the power on color C. sm1751x, sm1752x, sm17500 and hi512 have this function. 0-8 indicates the power on Color: black, red, green, yellow, blue, purple, green, white, and white of the fourth channel. When the power on color value set by sm17500 is 1, it means that the control chip is sm16813, otherwise it is other chips; when the power on color value set by sm17500 is 4, DMX512 protocol is output, otherwise, the output returns to zero code protocol. When hi512 is 8, power on self-test is enabled.

IV. press the menu key again to set the starting point address E. after inputting the address, press the OK key. Please wait a few seconds to complete the encoding.

Then press the menu key to return to the main menu.

⑦ The seventh menu is counting points.

⑧ The eighth menu is to set the working mode. At present, there are four modes 0-4.

The default is 0, normal mode;

1 is synchronous mode. Several h802rs form a local area network through routers or switches and need to be synchronized. If one of them is set to 1, the synchronization signal is sent. Others are 0.

2 is the timing mode, and the schedule file needs to be placed in SD, and the controller will execute the specified tasks according to the schedule.

In this mode, press "OK" to enter the date time setting, which is year, month, day, hour, minute and second.

3 is the aisle induction mode, the first file is normal file, the second file is induction file, the third file is normal file, the fourth file is induction file, the fifth file is normal file, the sixth file is induction file, and so on, odd sequence number is normal file, even serial number is induction file. The lamp receiving the induction signal plays the induction file, and other lights play the normal file.

In this mode, press "OK" to set the number of seconds after leaving, the number of points to install a sensor at port 1, and the number of points between port 2 to install a sensor. The sensor of port 2 is connected after the sensor of port 1, port 3 and port 2 share the same sensor, and port 4 and port 2 share the same sensor.

4 is the aisle induction mode, the first file is forward induction file, the second file is reverse induction file, the third file is forward induction file, the fourth file is reverse induction file, the fifth file is forward induction file, the sixth file is reverse induction file; ... and so on, odd numbers are positive induction files, even numbers are reverse induction files.

In this mode, press "OK" to set the number of sensors and the number of frames corresponding to a sensor.

⑨ The ninth menu is to set the playback mode to play all single file play sing. Press the "+" and "-" keys to switch to the mode. When accessing the console, you must select a single file to play.

5. Set the password. In the light modeling window of LED programming software, click the menu settings controller password to pop up the encryption dialog box. Select the controller encryption. The encryption method is to limit the power on times. After the first encryption, the second encryption can be performed without decryption to extend the usage times.

Ⅴ. Add and delete dat file

1. Connect both to the same router or computer. Note that the IP address of h802rs cannot conflict with other devices in LAN.

Start "led programming software", click "tools" menu - "send file", the software will automatically search the controller and display the file name in SD card. Select a file and click "delete" to delete the file.

2. Click "send dat file" to open the dialog box. Select the dat file to be added and click "open". The progress bar will display the sending progress. After completion, it will prompt that the sending is successful. Click "OK" and the list will refresh the file just sent.

ⅤI. Firmware upgrade

First connect h802rs to the Internet, press and hold the "menu" button and power on again. The controller will automatically upgrade. After the upgrade is completed, it will restart automatically and the version number will also change.

7、 Nixie tube status prompt

1. When power on, "h001" is the firmware version number of the controller, the display of "full" indicates that the encryption of the controller has been set and the use times have expired, and the display of "AC" indicates that the power synchronization function is started.

2. When reading the SD card, the display of "e FF" indicates that the dat file type is not correct, the display of "e Pb" indicates that there are too many points or more than one sub control number, the display of "SD" and flashing indicates that the SD card is reread, and the display of "e Fs" indicates that the SD card needs to be formatted.

3. The display of "saue" indicates that the parameters are being saved, the display of "---" indicates that it is in the online state, and the dot under the fourth nixie tube flashes to indicate that the online data has been received.

4. When adding or deleting files through LAN, the display of "del" indicates that the file is being deleted, the display of "D --" indicates that the content of the file is ready to be received. The number displayed in the receiving process is the number of K bytes left, the display of "succ" indicates the success of adding or deleting the file, and the display of "fail" indicates the failure of adding or deleting the file.

5. The display of "E -" indicates that the address code is ready, that "E --" indicates that the address code is being coded, and that "E --" indicates that the address code is completed.

ⅤII. Controller interface definition:

| GND DAT1 | GND DAT2 | GND DAT3 | GND DAT4 |
| --- | --- | --- | --- |
| GND D- 1 D+1 | GND D- 2 D+2 | GND D- 3 D+3 | GND D- 4 D+4 |